Fimco Pressurized 30-gal Sprayer with Mixer (LG-30-SM) WO#771119

Equipment

- Tank, 15’ pressure hose, and spray gun purchased in 2021

- Additional 100-ft pressure hose purchased in 2018

- Repair and spare part kits (contains pump, replacement gasket kit, washers, battery connectors, Teflon tape, fuses, multi-tool, hose clamps, socket driver, etc)

- 12-v automotive tractor or marine type battery and battery charger

- Boom Bucket and/or PVC Boom

Operation

- Charge 12-v battery before operating spray tank.

- Before filling tank, check the tank condition.

  - Is the tank clean? No visible cracks?

  - Is the hose in good condition? No visible cracks?

  - Check hose filter at bottom of tank to ensure it is clean.

    - Filter is inline after a value regulating flow from the tank to the pump, mixer and spray handgun or boom.

    - Use small flat head screwdriver to carefully (no nicks) pry filter from hose.

    - Fitting is tight, will require some force to reseat.

    - Make sure the value is ‘open’ or turned to be in-line with hose prior to turning on mixer or pump will not get fluid and may squeal or make other noises.

- Ensure drain cap is securely closed on tank.

- Attached hose to fitting label ‘Handgun’ If using the handgun to administer chemical. Make sure there is a washer in the fitting.

  - Extra washers are in the Tank Spare Part Box (clear with white lid).

    - Tee Jet CP19438-EPR gasket

  - Fitting is very tight, so it will take some strength to close value.

- Fill tank one-third to three quarters full of water (dependent on type and volume of pesticide used).

  - Add chemical.

  - Fill with water to volume of dilution or stock concentration.

- To mix chemical prior to delivery, attached battery wiring harness to tank pump.

  - Connect the end of the lead wire to the end of the pump.

  - The lead wire has an On/Off switch to activate the pump.

  - “1 or -” is ‘on’ and “O” is ‘off’.

- Connect battery clips on wiring harness to battery.

  - First connect red clip to the positive (red) terminal first.

  - Then connect black clip to the negative (black) connection.

  - Turn switch to ‘on’ position when ready to mix chemical with tank water.

- If pump sounds quite or not operating at preferred performance, Check battery or replace with fresh battery.

- To mix chemical, open ‘Pressure Adj.’ value to the level of mixing you desire.

- If mixer does not come on, check wiring harness fuses (there are two fuses).

  - Extra fuses are in the Tank Spare Part Box (clear with white lid).

  - After about 10–15 min of mixing, prime the tank hose into the pressurized tank.

    - Value setting is the same.

- To adjust pressure, use the far-left valve marked “Pressure Adj.” and slowly adjust to the pressure you desire.

- To deliver chemical without mixing, not usually done, then value settings would be:

  - This valve setting closes both valves

- Steps from #3-10 can be followed when using the Boom bucket to apply chemical

  - Agitator half open with pressure at 10 psi going out the Boom Bucket or PVC Boom

  - Agitator closed with pressure at 40 psi coming out the Bucket or PVC Boom

- If pond or clean water was used to pressurize hose to check for leaks, hose needs to be purged before applying chemical solution into burrow.

  - Clean water entering pond not as critical due to the volume being put into pond to reach nominal (target) concentration.

  - Tank concentration for burrow treatments = nominal concentration. If clean water is in line, crayfish and or burrow will be undertreated.

- When the tank is drained or empty, turn off pump via power switch.

  - Leave valves open and pull gun trigger to release pressure.

- Disconnect sprayer cables from battery.

  - Disconnect the negative (black) connection, then and the positive (red) connection from the battery, and finally disconnect the end of the pump cables from the lead wiring harness.

- Tank and hose should be cleaned after each use (tripled rinse with clean water).

  - For long-term storage, consider treating with liquid soap + 3x water rinse (see Tank Manual).

- Disconnect pressure hose and drain cap to allow water to drain from system (tank and tubing, hose, and booms).

| Study | Chemical Used | Type of chemical | Date | Est. Hours of Operation | Tank Rinsed after Use | Screen Cleaned After Use |
| --- | --- | --- | --- | --- | --- | --- |
| Michigan DNR-invasive crayfish control | ExciteR | Pyrethrin pesticide with petroleum distillates | Jul-21 | 50 | ? | yes |
| Michigan Prep | Well Water | N/A | Mar-23 | 0.5 | Yes | No |
| Michigan Prep | Well Water | N/A | April 12 | 1.0 | Yes | Cleaned Before Use |