Procedures for applying pesticide formulation to retention ponds

Treatment equipment

- Contaminant pads

- Warning signs

- Pesticide formulation

- Chemical mixing sheets

- Balance

- Plastic cylinders [for <1 container volume of pesticide]

- 1 jon boat (with load rating of at least 750 pounds) or bucket boom

- 1 trolling motor (set at 12” depth)

- 2 batteries (trolling motor, pressurized tank)

- 5-gallon buckets

- 1 30-gal pressurized sprayer tank

- 1 PVC boom-in-basket or pontoon boom attached to 150’ sprayer hose or PVC boom attached to Z Boat

- 4 5-gal carboy of well or tap water for pressurized tank rinsing

- 1 3-gal sprayer

- 2-3 100-150’ lengths of ¼” braided nylon rope

- Decontaminant tank

- Generator and gas [to run pump]

- Scrub brushes

- Nitrile or rubber gloves

- Rubber boots

- Apron or Tyvek coverall

- Eye protection (safety goggles, face shield) and eye wash bottle

- Absorbent or kitty litter, shovel, and trash bags (MDNR)

- Safety Station (see 2022 MDNR_Chemical Treatment_Safety station pump SOP)

Personal Protection Equipment (PPE) procedures

- Personnel should wear nitrile or rubber gloves, rubber boots or waders, eye protection, and apron as a barrier to concentrated pesticide.

- PPE should be changed between sampling sites.

- If traveling between sites, PPE should be stored in plastic or rubber containers in pick-up bed [not in passenger space].

- Disposable PPE can be disposed as residential trash.

- After use, non-disposable PPE (e.g., rubber boots or waders) should be rinsed with tap water to remove sediment from equipment then triple rinsed with well or tap water.

- If contaminated, clothing should be laundered separately with heavy-duty laundry soap twice in hot water.  A third empty cycle in hot water should be run thru machine after clothes have been cleaned. If clothing is completely saturated in pesticide, do not launder; double-bag and dispose of clothing.

Chemical mixing for large ponds

- Fill 30-gal pressurized tank one-third full, 4 5-gal carboys and 1 sprayer with tap or well water [may be done day before treatment].

- Set-up booms or Z Boat at edge of pond water.

- Set-up containment equipment.

- Place warning signs around pond to be treated.

- Review chemical mixing sheet for the volume of water and pesticide to treat pond.

- Transport pesticide containers to containment pad in 5-gal bucket to mixing station.

- Pour volume of pesticide needed to treat pond into pressurized tank.

- Tripe rinse each pesticide container into the pressurized tank.

- Fill tank with tap or well water to total treatment volume.

- Close lid and attach tank to battery to initiate pressurization and keep solution agitated. Close value for hose and turn-on pump to agitate solution for about 10-30 minutes prior to initiating chemical application.

- Begin treatment of ponds as soon as solution is well-mixed in tank.

Chemical delivery for ponds using Bucket or Pontoon boom or Z Boat

- Review ‘flight plan’ or delivery path for pond.

- Position treatment crew around pond.  Crew members should have rope to pull boom basket or be positioned at tank and be ready to deploy 100-150’ hose.

- Crew supervising chemical delivery should notify flight captain when ready for treatment to begin.

- Launch boat or boom.

- When crew ready, one crew member will open value of tank.

- Second crew member will observe solution flowing out of tank before dropping bucket into pond.

- Follow flight plan to deliver chemical.

- When pressure drops due to emptying of tank, turn off tank.

- Remove lid from tank.

- Using sprayer, rinse sides of tank with tap or well water.

- Fill tank with 5-gal of tap or well water using one of 5-gal carboys.

- Follow flight plan to deliver rinsate.

- Repeat tank draining procedure, rinse procedure, and rinsate delivery two additional times.

- Decontaminate equipment, as needed.

Sampling gear clean-up between sampling locations and for gear storage

- Rinse sampling gear with well or tap water in pond or on containment pond, brushing off all sediment particles or debris. Triple rinse with well or tap water.

- Air dry.  Place in sun for 24 hrs after use.

- Store cleaned equipment as directed by manuals.

Transfer of data

- Document field operations associated with sample collection on field sampling sheets or field notebooks.

- Scan data and email or mail data sheets/notebooks to CERC.

- Transfer any digital photographs of the sample sites and collection.

Trolling motor and battery

Boat treatment rig

Pressurized tank and battery; carboys and sprayer for rinsing tank

Boom for boat rig

Boom-in-a-bucket

Containment Pad- Large ponds

Containment Pad- Small ponds